Source: http://www.takakaira.co.jp/performance/defi/dl/bfmeter/bftroubleshoot.htm

Trouble Shooting

CONDITION

*The gauge shows low value in general.
*When ignition is on, the zero point slips out of position.

POSSIBLE CAUSE

*The ground wire is not properly fixed on the earth.
*Any harness wire is not installed properly.

REMEDY

*Extend the ground wire of the power wire and connect it properly to the battery minus terminal.
*Check wiring.

CONDITION

*The warning LED lamp flashes ON at a value different from the set warning value.
*The pointer is showing an abnormal value.

POSSIBLE CAUSE

*Various harnesses are picking up noise from spark signal or wireless radios, etc.

REMEDY

*Reposition the signal wire avoiding noise.

CONDITION

*The warning LED lamp is flashing ON and OFF.

POSSIBLE CAUSE

*There is a wire short circuit somewhere.
*There is a wire broken, or disconnected.
*Set the wrong cylinders number.

REMEDY

*Confirm that no wires are touching the vehicle body.
*Confirm that there are no wire broken or the connectors are not disconnected.
*Confirm that the setting of cylinders number is correct.

CONDITION

*Illumination of Defi-Link Meters doesn't go out being interlocked with the illumination switch off.

POSSIBLE CAUSE

*Defi-Link Meters are connected to METER OUTPUT2.

REMEDY

*Connect Defi-Link Meters to METER OUTPUT1.

CONDITION

*Brightness of Defi-Link Meter BF's illumination can not be adjusted.
*Illumination of Defi-Link Meter BF doesn't go on.

POSSIBLE CAUSE

*Defi-Link Meter BFs are connected to METER OUTPUT1.

REMEDY

*Connect Defi-Link Meters to METER OUTPUT2.

CONDITION

*The warning LED of oil pressure meter is lighted and a buzzer sounds when the engine is off.

POSSIBLE CAUSE

*The condition is normal.

REMEDY

*Oil Pressure is 0kPa when the engine is off and the warning LED is lighted and a buzzer sounds.